Secure IIoT-Based SCADA Motor Temperature Monitoring and Alert System

1. Introduction

Industrial Internet of Things (IIoT) plays a crucial role in modern industries by enabling real-time monitoring and automation. This project demonstrates a simulated SCADA-based motor monitoring system using Modbus protocol, where temperature is continuously observed and alerts are generated when unsafe conditions occur.

2. Objective

The main objectives of this project are:

- To monitor motor temperature using Modbus protocol

- To visualize real-time data using Node-RED dashboard

- To generate email alerts when temperature exceeds 70°C

- To implement cybersecurity concepts in IIoT systems

3. Tools and Technologies Used

- Node-RED

- Modbus TCP Server (Python)

- SCADA Demo Machine Interface

- Gmail SMTP (Email Alert)

- Kali Linux

4. System Architecture

SCADA Motor → Modbus Server → Node-RED → Dashboard + Alert System → Email Notification

5. Methodology

Step 1: Modbus Server Simulation

A Python-based Modbus TCP server was used to simulate motor temperature values ranging from 20°C to 70°C.

Step 2: Data Acquisition

Node-RED reads temperature values from the Modbus server using Modbus Read node.

Step 3: Data Processing

The received data is converted into numerical format using a function node.

Step 4: Condition Monitoring

A threshold condition is applied:

- If temperature ≥ 70°C → trigger alert through email

Step 5: Alert System

An email is sent automatically using SMTP when the threshold is crossed.

Step 6: Dashboard Visualization

Temperature values are displayed in real-time using a gauge in Node-RED dashboard.

6. Node-RED Flow Explanation

The flow consists of the following components:

- Modbus Read Node → Reads temperature

- Function Node → Converts data format

- Switch/Logic Node → Checks condition (≥70°C)

- Email Node → Sends alert

- Dashboard Gauge → Displays temperature

7. Screenshots

Screenshot 1: SCADA Motor Interface

Screenshot 2: Node-RED Flow

Screenshot 3: Email Alert Received

Screenshot 4: Dashboard Temperature Gauge

Screenshot 5: Modbus Server Running

8. Alert Logic Implementation

To avoid repeated alerts, a state-based logic was implemented. The system sends an email only when the temperature crosses 70°C from a lower value, preventing multiple alerts.

9. Cybersecurity Implementation

The following security measures were considered:

- Secure email authentication using App Password

- Controlled access to Node-RED

- Monitoring abnormal conditions (temperature threshold)

- Prevention of alert flooding

10. Results

- Real-time monitoring of motor temperature achieved

- Alert successfully triggered when temperature ≥ 70°C

- Dashboard visualization working correctly

- No repeated alerts due to smart logic implementation

11. Advantages

- Real-time monitoring

- Automated alert system

- Easy to implement

- Scalable for industrial use

12. Limitations

- Simulation-based system (no real hardware)

- Dependent on network connectivity

- Basic security implementation

13. Future Scope

- Integration with real sensors (ESP8266/ESP32)

- SMS/Telegram alerts

- AI-based predictive maintenance

- Cloud integration (AWS/Azure IoT)

14. Conclusion

This project successfully demonstrates a secure IIoT-based monitoring system for a SCADA motor. The system efficiently detects high temperature conditions and generates alerts, ensuring safety and reliability in industrial environments.